SECTION 09 97 23

CONCRETE AND MASONRY COLOR TREATMENT

Display hidden notes to specifier. (Don't know how? Click Here)

** NOTE TO SPECIFIER ** Nawkaw Inc.; and NawTone stain product.

This section is based on the products of Nawkaw Inc., which is located at:

170 Whitetail Way

Bogart, GA 30622

Toll Free Tel: 866-462-9529

Tel: 706-355-3217

Email: info@nawkaw.com

Web: http://www.nawkaw.com/

In 1988, Nawkaw was founded in Toronto, Canada, by CEO, Russell Gray. Ever since, Nawkaw has pioneered the concrete and masonry industry by manufacturing innovative, high-quality, and environmentally-friendly products that contain little to zero VOC. Nawkaw offers both commercial and residential services and strives to invent techniques that result in outstanding finishes on concrete and masonry. By applying color and adding texture to your preferred building material, we can create any look you desire.

This specification includes Nawkaw's NawTone stain.

NawTone stain is in use on more buildings than any other masonry stain in the world. Since 1987, Nawkaw has had a track record of excellent performance, product testing, and innovation—setting the standard of excellence for the coatings industry.  Nawkaw installation teams have won multiple awards using NawTone.

PART  1 GENERAL

  - SUMMARY

- Section Includes: Water Based Acrylic Stain and coatings system applications. Specifications include limited service preparation.

- RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

  - Section 03 30 00 - Cast-in-Place Concrete.

  - Section 03 11 16.13 - Concrete Form Liners.

  - Section 03 35 23 - Exposed Aggregate Concrete Finishing.

  - Section 03 41 16 - Precast Concrete Slabs.

  - Section 03 45 13 - Faced Architectural Precast Concrete.

  - Section 03 47 13 - Tilt-Up Concrete.

  - Section 03 49 00 - Glass-Fiber-Reinforced Concrete.

  - Section 03 50 00 - Cast Decks and Underlayment.

  - Section 04 20 00 - Unit Masonry: Concrete Masonry Units.

  - Section 04 21 13- Brick Masonry.

  - Section 04 40 00 - Stone Assemblies.

  - Section 09 22 00 - Stucco.

  - Section 09 24 13 - Adobe Finish.

- Related Products [DELETE ARTICLE C. IN FINAL SPECIFICATION IF NECESSARY]

  - Primers

    - Nawkaw NawThinz-C

    - Nawkaw NawThinz-M

    - Nawkaw NawTone-K

    - Nawkaw NawTone-K Opaque

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

- ASTM Standards

  - ASTM C 744 - Standard Specification for Prefaced Concrete and Calcium Silicate Masonry Units.

  - ASTM C 216 – Standard Specification for Facing Brick (Solid Masonry Units Made from Clay or Shale)

  - ASTM E 96 – Standard Test Methods for Water Vapor Transmission of Materials

  - ASTM E 514 – Standard Test Method for Water Penetration and Leakage Through Masonry

  - ASTM G 154 – Standard Practice for Operating Fluorescent Light Apparatus for UV Exposure of Nonmetallic Materials

- SCAQMDR 1168 – South Coast Air Quality Management District's (SCAQMD) Volatile Organic Compounds (VOC) Rule 1168.

- ASN/ZS 1530.3-1999 – Methods for Fire Tests on Building Materials, Components and Structures

- AS/NZS 4456.10 – Masonry units, segmental pavers and flags-Methods of test

Method 10: Determining resistance to salt attack

  - SYSTEM DESCRIPTION

- A materials-compatible highly vapor permeable water-repelling and weather resistant decorative acrylic stain system

  - Acrylic Stain: single coat system occasionally comprising of a primer/basecoat depending on the finish desired.

  - Acrylic Stain penetrates and creates a superior mechanical bond with the substrates surface.

1.4 	SUBMITTALS

A. 	Submit under provisions of Section 01 30 00 - Administrative Requirements.

B. 	Product Data: Manufacturer's data sheets on each product to be used, including:

1. 	Product characteristics.

2. 	Preparation instructions and recommendations.

3. 	Storage and handling requirements and recommendations.

4. 	Installation methods.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected and specified herein or indicated on the drawings.

C. 	Preliminary Samples:  To be provided as required for the specific project.

D. 	Verification Samples:   To be provided on the specific materials to be treated when they are available in plant or on site.

E. 	Manufacturer's Certificates: Certify products meet or exceed specified requirements.

1.5 	QUALITY ASSURANCE

A. 	Manufacturer Qualifications: An international manufacturer with a minimum of 20 years of experience in the production of the stains and coatings of type specified.

B. 	Installer Qualifications: Installer licensed by Nawkaw to apply the stain products specified and with a minimum of three years documented experience in applying stains and coatings similar in type and scale to this Project.

C.	Environmental Regulations: The masonry stain material to be applied is in compliance with federal, provincial and local environmental Volatile Organic Compounds (VOC) regulations.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

D. 	Mock-Up: Apply a minimum one square foot sample of each type of color application required.

1. 	Finish areas designated by Architect.

2. 	Prepare each sample in an area where it will be exposed to the same conditions as will be present on the building during curing.

3. 	Samples should be viewed from a minimum distance of 20 feet.

4. 	Do not proceed with remaining work until color and finish is approved by Architect.

5.	Maintain record of the approved mockup stain color, application ratio and finish to ensure color consistency and appearance aesthetics.

1.6 	DELIVERY, STORAGE, AND HANDLING

A. 	Store products in manufacturer's unopened packaging until ready for installation.

B. 	Store and handle products in accordance with requirements of manufacturer.

C. 	Store materials inside if possible, away from open flame. Store in a secure area to avoid tampering and contamination. Water-based materials must be kept from freezing.

1.7 	PROJECT CONDITIONS

A. 	Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

1.8 	WARRANTY

A. 	At project closeout, provide to Owner or Owner’s Representative an executed copy of the manufacturer’s standard limited warranty against manufacturing defects, outlining its terms, conditions, and exclusions from coverage.

1.	 Duration:

- Color treatment: 25 years.

- Water Repellency: 5 years

PART  2 PRODUCTS

2.1 	MANUFACTURERS

- Basis of Design:

1.	Items specified are to establish a standard of quality for design, function, materials, compatibility, performance, warranty, and appearance.

** NOTE TO SPECIFIER ** Delete one or two of the following paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

2.	Equivalent products by listed manufacturers are acceptable.

3.	Substitutions: Not permitted.

4.	Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

- Acceptable Manufacturer:

  - Nawkaw Inc., location: 170 Whitetail Way, Bogart, GA 30622.

Toll Free Tel: 866-462-9529; Tel: 706-355-3217;

Email: info@nawkaw.com; Web: http://www.nawkaw.com/

2.2 	MATERIALS

** NOTE TO SPECIFIER ** NawTone is a hydrous, specially formulated base containing high-quality polymers. The addition of lightfast pigments enables a wide range of colors to be produced. NawTone exhibits the overall characteristics of being: lightfast, UV resistant, penetrating, aspiratory, quick-drying, and durable, while being mold, mildew and weather resistant.

A. 	Water-Based Stain

1. 	Basis of Design: NawTone (Formerly NECT-90) is a high-quality, water-based, highly permeable acrylic stain. Mold, mildew, UV and weather resistant.

2. 	Properties:

• Viscosity: (72° F) 70°-90° KU

• pH: 8.5-9.5

• Finish: flat

• ASN/ZS 1530.3-1999: Nonflammable

• SCAQMDR 1168VOC: < 5 g/L

• Abrasion Resistance: excellent

• ASTM C 216, Freeze/Thaw Test: exceeded

• AS/NZS 4456.10, Salt Attack Resistance: no blisters

• ASTM E 514, no water appearance, no dampness, no water collection

• ASTM E 96, 	Water Vapor Transmission: 0.337 g/hr m2

Water Vapor Permeance: 6.6x10-8 g/Pa s m2

•ASTM (G 154, G 53, D2244) UV Resistant–Accelerated Weathering, 2000 hrs: excellent

3. 	Finish:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete the one not required.

a. 	Color: To match existing substrate (masonry or concrete).

b. 	Color: Formulate to color as selected by the Owner or Architect.

PART 3 EXECUTION

3.1 	EXAMINATION

A. 	Verification of Conditions: Confirm by examination the areas and conditions under which the work is to be applied for compliance with manufacturer’s instructions. Do not begin installation until substrates have been properly prepared and unsatisfactory conditions have been rectified.

** NOTE TO SPECIFIER ** Select one or more of the following paragraphs as applicable. Delete if not applicable.

- Verify that new masonry and concrete have cured at least 21 days prior to applying NawTone.

- Verify substrate has no pretreatments or priming materials applied unless such conditions are approved by manufacturer.

- Verify that surfaces being color treated with NawTone have a neutral pH, are clean, dry and free of efflorescence.

- If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

3.2 	PREPARATION

A. 	Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

- Following either sand-blasting or light duty concrete cleaner wash, standard pressure washing protocol for cleaning should be followed and the surface should then be allowed to dry to specified conditions by the manufacturer.

B. 	Clean surfaces thoroughly prior to installation. Allow surfaces to dry completely before applying coating.

C. 	Verify that walls, masonry, concrete, stucco, block split faced/fluted and mortar that may have been treated with any form of chemical/acid wash are neutralized.

D. 	Treat alkali or efflorescence with proper neutralizing compounds as recommended by masonry supplier before stain application.

** NOTE TO SPECIFIER ** Note that a neutral pH is required to maintain the manufacturer's warranty.

E. 	Before application, verify that the masonry walls have a neutral pH.

F. 	Before application, verify that surface to be treated is clean, dry and contains no frozen water.

G. 	Mix products as recommended immediately prior to application.

3.3 	INSTALLATION

A. 	Install in accordance with manufacturer's instructions.

B. 	Apply stain using airless spray pump to help control airborne particles or overspray. If site conditions prohibit spray application, apply by hand; utilizing brushes and rollers.

C. 	Do not proceed with work when ambient temperatures are less than 25 degrees F (-4 degrees C) or greater than 110 degrees F (43 degrees C).

D.	Allow manufacturer's specified drying time for each coat before applying next coat (if required).

E. 	Verify color consistency. Recoat areas where blotches, blemishes or imperfections are present.

3.4 	FIELD QUALITY CONTROL

A. 	Verify color consistency. Recoat any areas that are unacceptable.

3.5 	PROTECTION

A. 	Protect installed products until completion of project.

B. 	Protect prefinished items, concealed surfaces, finished metal surfaces, operating parts, and labels as required.

C. 	Protect shrubs, metal, wood trim, glass, asphalt and other building hardware during application from overspray.

D. 	Do not permit mist (if spraying) or liquid to drift onto surrounding properties or parking lots.

E. 	Touch-up, repair or replace damaged products before substantial completion.

END OF SECTION